| Name: | _Muhammad_Usman__ |  |  | Roll number: |  | _23BSCYS006___ |
| --- | --- | --- | --- | --- | --- | --- |
| Score: | __________ | Date: | ____________ | Instructor’s Signature: |  | ______________ |

# Centralized WLAN Management Using Huawei Access Controller with Cloud-Based GUI Access

Rubrics

| LAB PERFORMANCE INDICATOR | SUBJECT KNOWLEDGE | DATA ANALYSIS AND INTERPRETATION | ABILITY TO CONDUCT EXPERIMENT |
| --- | --- | --- | --- |
| SCORE |  |  |  |

Note: Student performance will be assessed based on this rubric during lab sessions.

Introduction

Wireless networks have become essential for modern connectivity, enabling mobility and flexibility across various environments. Centralized wireless management through Access Controllers (ACs) is critical for organizations requiring scalable, secure, and manageable WLAN deployments.

This project focuses on the implementation and configuration of a Huawei Access Controller (AC-1) for centralized wireless network management. The AC serves as the primary control point for multiple Access Points (APs), providing enterprise-grade WLAN services with secure authentication.

A key feature of this implementation is the integration of Cloud connectivity within the eNSP simulation environment, enabling GUI-based (Graphical User Interface) management of the AC. This approach demonstrates practical wireless network management skills essential for HCIA-Datacom certification.

The Access Controller manages two Access Points (AP1 and AP-2) that broadcast the "HCIA" SSID with WPA2-PSK security, providing wireless connectivity to end-users while maintaining separate management traffic.

Project Description

1. Project Objectives

- Configure AC-1 as the centralized WLAN controller

- Establish CAPWAP tunnels between AC and multiple Access Points

- Implement secure wireless access using WPA2-PSK with AES encryption

- Enable GUI-based management through Cloud integration in eNSP

- Verify AP registration and wireless client connectivity

- Document all configuration steps with verification commands

2. Scope of Work

In Scope:

- AC-1 basic configuration (system name, VLANs, interfaces)

- DHCP pool configuration for AP address assignment

- CAPWAP source interface configuration

- WLAN service configuration (SSID profile, Security profile, VAP profile)

- AP group creation and AP registration

- Cloud device configuration for GUI access

- Connectivity and functionality testing

Out of Scope:

- RADIUS-based enterprise authentication (802.1X)

- Advanced roaming configurations

- RF management and RRM optimization

3. Device Roles

| Device | Hostname | Role | IP Address | VLAN |
| --- | --- | --- | --- | --- |
| AC-1 | AC-1 | Wireless Controller | 192.168.100.254 | VLAN 100 |
| SW-1 | SW-1 | Distribution Switch | 192.168.101.254 | VLAN 100, 101 |
| AP1 | AP1 | Access Point | DHCP (192.168.100.2) | VLAN 100 |
| AP-2 | AP-2 | Access Point | DHCP (192.168.100.3) | VLAN 100 |

4. IP Addressing Scheme

| Network | Subnet | Gateway | Purpose |
| --- | --- | --- | --- |
| AP Management | 192.168.100.0/24 | 192.168.100.254 | CAPWAP tunnels, AP registration |
| Wireless Client | 192.168.101.0/24 | 192.168.101.254 | End-user wireless devices |

Methodology

1. Design Process

The implementation followed a systematic approach:

Phase 1: Requirements Analysis

- Identified need for centralized wireless management

- Defined VLAN segmentation (Management VLAN 100, Client VLAN 101)

- Determined security requirements (WPA2-PSK with AES)

Phase 2: Architecture Design

- Designed AC-AP communication using CAPWAP protocol

- Planned trunk ports for carrying multiple VLANs

- Designed DHCP infrastructure for AP addressing

Phase 3: Configuration Planning

- Mapped configuration hierarchy: System → VLAN → DHCP → CAPWAP → WLAN Profiles → AP Registration

Phase 4: Implementation & Testing

- Executed configurations on AC-1 and supporting switches

- Verified AP registration and CAPWAP tunnel status

- Tested client connectivity and GUI access

2. Implementation Technologies

CAPWAP (Control and Provisioning of Wireless Access Points)

- Protocol used for AC-AP communication

- Separates control plane (AC) from data plane (AP)

- Enables centralized configuration management

VLAN Segmentation

- VLAN 100: AP management and CAPWAP control traffic

- VLAN 101: Wireless client data traffic

DHCP (Dynamic Host Configuration Protocol)

- AC-1 acts as DHCP server for APs on VLAN 100

- SW-1 acts as DHCP server for wireless clients on VLAN 101

WPA2-PSK Security

- Pre-Shared Key authentication

- AES encryption for data confidentiality

3. Configuration Steps

Step 1: Basic System Configuration on AC-1

bash

sysname AC-1

vlan batch 100 to 101

Step 2: DHCP Pool Configuration for APs

bash

dhcp enable

ip pool ap

gateway-list 192.168.100.254

network 192.168.100.0 mask 255.255.255.0

Step 3: Interface Configuration

bash

interface Vlanif100

ip address 192.168.100.254 255.255.255.0

dhcp select global

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 100 to 101

Step 4: CAPWAP Source Configuration

bash

capwap source interface vlanif100

Step 5: WLAN Profile Configuration

bash

# SSID Profile

ssid-profile name HCIA

ssid HCIA

# Security Profile (WPA2-PSK)

security-profile name HCIA

security wpa-wpa2 psk pass-phrase %^%#@'8rUt5/wH+aEf~BI8|1m-v&*~IiCPjwI':}n,>@%^%# aes

# VAP Profile

vap-profile name HCIA

service-vlan vlan-id 101

ssid-profile HCIA

security-profile HCIA

Step 6: AP Group and AP Registration

bash

# Create AP Group

ap-group name ap-group1

radio 0

vap-profile HCIA wlan 1

radio 1

vap-profile HCIA wlan 1

radio 2

vap-profile HCIA wlan 1

# Register AP1

ap-id 0 type-id 61 ap-mac 00e0-fc9c-50a0 ap-sn 2102354483109D72487A

ap-name AP1

ap-group ap-group1

# Register AP-2

ap-id 1 type-id 61 ap-mac 00e0-fc5b-6890 ap-sn 2102354483108467096B

ap-name AP-2

ap-group ap-group1

Step 7: HTTP Service for GUI Access

bash

local-user admin password irreversible-cipher $1a$:_vq4gDWJ)$c7+CAzgKlNe*m7A{/d](K;{HVn5V@O/,)6Rz!Y/%$

local-user admin privilege level 15

local-user admin service-type http

4. Cloud Integration for GUI Access

To access the AC GUI via Cloud in eNSP:

Procedure:

- Add Cloud device to eNSP topology

- Configure UDP port mapping between eNSP and host machine

- Map local port 8080 to AC's HTTP port 80

- Access http://192.168.100.254 from host browser

eNSP Simulation (Configuration File, Simulation/Results)

1. Simulation Environment Setup

The simulation was performed using Huawei eNSP (Enterprise Network Simulation Platform) . The topology includes:

- 1 Access Controller (AC-1)

- 1 Distribution Switch (SW-1)

- 2 Access Points (AP1, AP-2)

- 1 Cloud device for GUI access

2. Configuration Files

AC.cfg - Complete AC Configuration:

text

[V200R007C10SPC300]

#

sysname AC-1

#

vlan batch 100 to 101

#

dhcp enable

#

ip pool ap

gateway-list 192.168.100.254

network 192.168.100.0 mask 255.255.255.0

#

interface Vlanif100

ip address 192.168.100.254 255.255.255.0

dhcp select global

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 100 to 101

#

capwap source interface vlanif100

#

wlan

ssid-profile name HCIA

ssid HCIA

security-profile name HCIA

security wpa-wpa2 psk pass-phrase %^%#@'8rUt5/wH+aEf~BI8|1m-v&*~IiCPjwI':}n,>@%^%# aes

vap-profile name HCIA

service-vlan vlan-id 101

ssid-profile HCIA

security-profile HCIA

ap-group name ap-group1

radio 0

vap-profile HCIA wlan 1

radio 1

vap-profile HCIA wlan 1

radio 2

vap-profile HCIA wlan 1

ap-id 0 type-id 61 ap-mac 00e0-fc9c-50a0 ap-sn 2102354483109D72487A

ap-name AP1

ap-group ap-group1

ap-id 1 type-id 61 ap-mac 00e0-fc5b-6890 ap-sn 2102354483108467096B

ap-name AP-2

ap-group ap-group1

#

return

SW-1.cfg - Distribution Switch Configuration:

text

#

sysname SW-1

#

vlan batch 100 to 101

#

dhcp enable

#

ip pool sta

gateway-list 192.168.101.254

network 192.168.101.0 mask 255.255.255.0

#

interface Vlanif101

ip address 192.168.101.254 255.255.255.0

dhcp select global

#

interface GigabitEthernet0/0/1

port link-type trunk

port trunk allow-pass vlan 100 to 101

#

interface GigabitEthernet0/0/2

port link-type trunk

port trunk allow-pass vlan 100 to 101

#

interface GigabitEthernet0/0/3

port link-type trunk

port trunk allow-pass vlan 100 to 101

#

return

3. Verification Commands and Outputs

Verify AP Registration:

text

<AC-1> display ap all

Total AP information:

nor  : normal          [2]

----------------------------------------------------

ID   MAC            Name   Group     IP             State

0    00e0-fc9c-50a0 AP1    ap-group1 192.168.100.2  normal

1    00e0-fc5b-6890 AP-2   ap-group1 192.168.100.3  normal

Verify VAP Status:

text

<AC-1> display vap all

AP ID  Radio ID  VAP ID  SSID   Status  Auth Type    VLAN

0      0         1       HCIA   ON      WPA2-PSK     101

0      1         1       HCIA   ON      WPA2-PSK     101

0      2         1       HCIA   ON      WPA2-PSK     101

1      0         1       HCIA   ON      WPA2-PSK     101

1      1         1       HCIA   ON      WPA2-PSK     101

1      2         1       HCIA   ON      WPA2-PSK     101

Verify CAPWAP Tunnel Status:

text

<AC-1> display capwap ap all

AP-ID  State       IP-Address       MAC-Address       Name

0      normal      192.168.100.2    00e0-fc9c-50a0    AP1

1      normal      192.168.100.3    00e0-fc5b-6890    AP-2

Verify DHCP Address Pool:

text

<AC-1> display ip pool ap used

Pool-Name      : ap

Gateway        : 192.168.100.254

Network        : 192.168.100.0

Mask           : 255.255.255.0

Used addresses : 2

Idle addresses : 251

Results

Description of Results

The simulation successfully validated all configuration objectives through comprehensive testing:

Test 1: AP Registration

- Test: Verify AP1 and AP-2 register with AC-1

- Expected: Both APs show "normal" state

- Result: ✓ PASS - Both APs successfully registered

Test 2: DHCP Address Assignment for APs

- Test: Verify APs receive IP addresses from AP pool

- Expected: IPs in 192.168.100.0/24 range

- Result: ✓ PASS - AP1: 192.168.100.2, AP-2: 192.168.100.3

Test 3: SSID Broadcast

- Test: Verify "HCIA" SSID is broadcast on all radios

- Expected: SSID visible on 2.4GHz and 5GHz bands

- Result: ✓ PASS - SSID active on all 6 radios (3 per AP)

Test 4: CAPWAP Tunnel Establishment

- Test: Verify CAPWAP tunnels between AC and APs

- Expected: State shows "normal" for both APs

- Result: ✓ PASS - Tunnels established successfully

Test 5: Client DHCP Address Assignment

- Test: Wireless client connects and receives IP address

- Expected: Client receives IP from VLAN 101 (192.168.101.x)

- Result: ✓ PASS - Client successfully obtains IP address

Test 6: GUI Access via Cloud

- Test: Access AC web interface through Cloud device

- Expected: Login page displays and authentication succeeds

- Result: ✓ PASS - GUI accessible at http://192.168.100.254

Testing Summary

| Test Case | Status | Remarks |
| --- | --- | --- |
| AP Registration | ✓ PASS | Both APs registered |
| DHCP for APs | ✓ PASS | IPs from 192.168.100.0/24 |
| CAPWAP Tunnel | ✓ PASS | Tunnels established |
| SSID Broadcast | ✓ PASS | "HCIA" visible |
| Client Connectivity | ✓ PASS | Successful association |
| Client DHCP | ✓ PASS | IP from 192.168.101.x |
| GUI Access (Direct) | ✓ PASS | HTTP access working |
| GUI Access (Cloud) | ✓ PASS | Port forwarding successful |

Overall Success Rate: 8/8 (100%)

Challenges Encountered

Challenge 1: Cloud Device Port Mapping

- Problem: Unable to access AC GUI through Cloud device

- Solution: Reconfigured Cloud device with proper inbound/outbound port mapping

Challenge 2: AP Registration Failure

- Problem: APs not registering with AC

- Solution: Verified capwap source interface vlanif100 and DHCP option 43

Challenge 3: VLAN Mismatch

- Problem: APs not communicating with AC

- Solution: Verified trunk port configuration on SW-1

Conclusion

This project successfully demonstrated the complete configuration and management of a Huawei Access Controller (AC-1) for enterprise wireless network deployment. All objectives were achieved:

- ✓ AC Configuration - Successfully configured AC-1 with VLANs 100 and 101

- ✓ WLAN Service Deployment - Created and deployed SSID "HCIA" with WPA2-PSK security

- ✓ AP Management - Registered two access points in ap-group1

- ✓ Cloud Integration - Established GUI access through eNSP Cloud device

- ✓ Client Connectivity - Verified wireless client association and DHCP